Q: CONSTRUCTION AND WORKING OF ULTRA SMALL ASYMMETRIC SUPERCAPACITOR

ANS:-Supercapacitor is an electrochemical energy storage device that stores energy through electrostatic double layer capacitance(EDLC) and electrochemical psuedocapacitance. Exihibiting high power density

Construction:

- Anode : carbon based nanomaterials like graphene

- Cathode: Transition metal oxide (Mn02) with polytetrafluroethylene and n-methyl pyrlodine

- Electrolyte and seperator: Flexible PVA gel membrane treated with aqueous H2SO4 acts both electrolyte and separator (PVA/H2SO4 gel membrane)

Working:

- During the charging and discharging process, the MnO2 positive electrode exhibits pseudocapacitance, storing energy through fast and reversible REDOX reactions.

- MnO2 undergoes a reversible transformation from its +4 oxidation state (MnO2) to +3 oxidation state (MnOOH) in the presence of an acidic electrolyte.

- M𝐧𝐎𝟐 + 𝐇+ + 𝐞− ⇌ 𝐌𝐧𝐎𝐎𝐇

- Meanwhile, the graphene negative electrode exhibits electric double-layer capacitance (EDLC) by adsorbing ions at its surface without undergoing any chemical change. The PVA/ H2SO4 gel membrane serves as a solid-state ionic conductor, enabling the transport of protons (H⁺) between the electrodes to maintain charge balance.

CONSTRUCTION AND WORKING OF SOLID OXIDE FULE CELL

- Cathode: Lanthanum strontium Maganite

- Anode: Nickel and Yttria stabilised Zirconia

- Electrolyte:Yttria stabilized Zttria stabilized Zirconia

Working:

- In Solid Oxide Fuel Cell (SOFC), ceramic components get activated at very high temperatures (~1000°C).

- Hydrogen (H2) fuel and oxygen (O2) as an oxidant are continuously fed into the inlets of the anode and cathode, respectively.

- At the anode, H2 undergoes oxidation, releasing H+ ions and e−.

- The e− flow through an external circuit, generating electricity, and enter the cathodic compartment.

- At the cathode, O2 picks up the e− and gets reduced to oxygen anions (O2−).

- The O2− anions then migrate through the solid oxide electrolyte to the anode, where they combine with H+ ions to produce water.

- Electrode reactions:

Reaction at anode  𝐇𝟐  𝟐𝐇+ + 𝟐𝐞− 2𝐇+ + 𝐎𝟐−  𝐇𝟐𝐎

- Reaction at cathode 𝐎𝟐+ 2𝐞−  𝐎𝟐−

- Overall reaction 𝐇𝟐 + ½ O2  𝐇𝟐𝐎